# Objective

The objective of this lab is verify Norton’s theorem and to find the full load current.

# Apparatus Required

| Sr. no | Apparatus | Range | Quantity |
| --- | --- | --- | --- |
| 1 | Regulated Power Supply | Standard | 1 |
| 2 | Signal Generator | Standard | 1 |
| 3 | Oscilloscope | Standard | 1 |
| 4 | Digital Multimeter | Standard | 1 |
| 5 | Breadboard & Wires | Standard | As Required |
| 6 | Resistors | 1k, 330 ohms | 1,3 |
| 7 | Potentiometer | Standard | 1 |

Description

Statement:

Any linear circuit can be simplified to an equivalent circuit consisting of a single current source and parallel resistance that is connected to a load.

Precautions:

- Voltage control knob of RPS should be kept at minimum position.

- Current control knob of RPS should be kept at maximum position

# Procedure for Norton’s Theorem:

- Connections are given as per circuit diagram.

- Set a particular value in RPS and note down the ammeter readings in the original circuit.

To Find IN:

- Remove the load resistance and short circuit the terminals.

- For the same RPS voltage note down the ammeter readings.

To Find RN:

- Remove RPS and short circuit the terminal and remove the load and note down the resistance across the two terminals.

Equivalent Circuit:

- Set IN and RN and note down the ammeter readings.

- Verify Norton’s theorem.

To find load current in circuit 1:

To find IN

To find RN

# Norton’s equivalent circuit

|  | E (volt) | IN (mA) | RN | IL (mA) |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Circuit - I | Equivalent Circuit |
| Practical Values | 10 | 10.1 | 4.91 | 3.34 | 3.34 |

Post Lab Questions:

1. What is Norton’s Theorem?
Norton’s Theorem states that any linear electrical circuit can be replaced by an equivalent circuit consisting of:

- A single current source (Norton current, In)

- A single parallel resistance (Norton resistance, Rn)

This makes complex circuit analysis easier.

2. Applications of Norton’s Theorem:

- Simplifies complex circuit analysis

- Helps calculate current through a load

- Used in electrical network design

- Useful for analysing different load conditions

- Helps in troubleshooting circuits

3. Theoretical and Practical Limitations of Norton’s Theorem:

Theoretical Limitations:

- Applicable only to linear circuits

- Cannot be directly applied to non-linear components like diodes and transistors

Practical Limitations:

- Difficult for very large circuits

- Requires extra calculations to find Norton current and resistance

- Becomes complex when dependent sources are present

These are short exam-style answers.

Proteus Diagram: