UNIVERSITY OF ENGINEERING AND TECHNOLOGY

LAHORE CAMPUS

Lab #2

Circuit Analysis Lab

PRESENTED BY:

Muhammad Ibrahim Kaleem – 2025-CE-103

PRESENTED TO:

Prof. Mehmood

COMPUTER ENGINEERING – SECTION B

# Circuit Analysis Lab #2

# Experiment #1:

Signal Generator & Oscilloscope:

- Use the signal generator and the oscilloscope to perform the following tasks.

- Build circuit ‘A’ shown below in Figure 1.

- Set the signal generator to generate a sinusoidal signal with a frequency of 100 Hz and peak-to peak voltage of 5V.

- Set up one probe across input signal, and another across R1 and R2 alternatively.

- Measure and draw all the voltages and time related parameters on the graphs below.

Explanation:

To set up the simulation for Experiment #1 (Circuit A) in Proteus, follow this concise hardware setup:

- Components: Place a Signal Generator, two resistors (R1 = 5.1kΩ, R2 =3.3kΩ) and a Digital Oscilloscope.

- Circuit Path: Connect the Signal Generator in series with R1 and R2. Attach a Ground terminal to the bottom of R2 and the negative side of the generator.

- Probing: Connect Oscilloscope Channel A to the top of R1 (measures the total Input Signal).

  - Connect Oscilloscope Channel B to the node between R1 and R2 (measures the signal across R2).

- Signal Settings: Set the Generator to Sine Wave, Frequency to 100 Hz, and Amplitude (AMPL) to 2.5V (which produces the required 5V Peak-to-Peak).

| Resistor | Input Voltage | RMS Voltage | P-P Voltage |
| --- | --- | --- | --- |
| R1 | 5V | 1.07V | 3.04V |
| R2 | 5V | 0.69V | 1.96V |

Working for P-P Voltage:

VR1(P-P) = 5V x (5.1k/8.4k) ≈ 3.04V (Vp = 3.04/2 = 1.52V)

VR2(P-P) = 5V x (3.3k/8.4k) ≈ 1.96V (Vp = 1.96/2 = 0.98V)

Working for RMS Voltage:

For R1: 1.52V x 0.707 = 1.07V

For R2: 0.98V x 0.707 = 0.69V

Figure 2: Input Voltage Signal

Scale: 1V/Div in x-axis & 1ms/div as frequency is 100 and time period is 10ms

Figure 3: Voltage Signal Across R1

Scale: 1V/Div in x-axis & 1ms/div as frequency is 100 and time period is 10ms

Figure 5: Voltage Signal Across R2

Scale: 1V/Div in x-axis & 1ms/div as frequency is 100 and time period is 10ms

Figure 6: Draw 7V Sinusoidal

Scale: 1V/Div in x-axis & 1ms/div as frequency is 100 and time period is 10ms

Circuit Diagram:

Digital Oscilloscope (Figure 2 & 3)

Digital Oscilloscope (Figure 5)

Digital Oscilloscope (Figure 6)

# Experiment #2:

Signal Generator & Oscilloscope:

- Use the signal generator and the oscilloscope to perform the following tasks.

- Build circuit ‘B’ shown below in Figure 7.

- Set the signal generator to generate a sinusoidal signal with a frequency of 200 Hz and peak-to peak voltage of 7V.

- Set up one probe across input signal, and another across R1, R2 and R3 alternatively.

- Measure and draw all the voltages and time related parameters on the graphs below

Explanation:

To set up the simulation for Experiment #2 (Circuit 'B') in Proteus, follow this short hardware configuration:

Circuit Setup

- Components: Place a Signal Generator, three resistors (R1 = 4.7kΩ, R2 = 3.3kΩ, R3 = 5.1kΩ) and a Digital Oscilloscope.

- Circuit Path: Connect the Signal Generator in series with R1, R2, R3 in a single loop. Attach a Ground terminal to the end of R3 and the negative side of the generator.

- Probing: Connect Channel A to the start of R1 for the total input signal.

  - Connect Channel B to the node between R1 and R2.

  - Connect Channel C to the node between R2 and R3.

- Signal Settings: Set the Generator to a Sine Wave, Frequency to 200 Hz, and Amplitude (AMPL) to 3.5V to achieve the required 7V Peak-to-Peak.

| Resistors | Input Voltage | RMS Voltage | P-P Voltage |
| --- | --- | --- | --- |
| R1 | 7V | 0.89V | 2.51V |
| R2 | 7V | 0.62V | 1.76V |
| R3 | 7V | 0.96V | 2.73V |
| R1 & R2 | 7V | 1.51V | 4.27V |
| R2 & R3 | 7V | 1.58V | 4.49V |

1. Circuit Parameters

- Total Resistance (Rtotal): 13.1kΩ

- Input Voltage (VP-P): 7V

- Input Peak Voltage (Vp): 7V/2 = 3.5V

2. Working for P-P Voltage (Vp-p)

We use the Voltage Divider Rule: VRx = Vtotal x (Rx / Rtotal).

- VR1(P-P): 7V x (4.7k / 13.1k) ≈ 2.51V

- VR2(P-P): 7V x (3.3k / 13.1k) ≈ 1.76V

- VR3(P-P): 7V x (5.1k / 13.1k) ≈ 2.73V

- VR1+R2(P-P): 7V x (8.0k / 13.1k) ≈ 4.27V

- VR2+R3(P-P): 7V x (8.4k / 13.1k) ≈ 4.49V

3. Working for RMS Voltage (Vrms)

First, find the individual Peak Voltage (Vp) for each, then multiply by 0.707.

- For R1: Vp = 2.51V / 2 = 1.255V x 0.707 ≈ 0.89V

- For R2: Vp = 1.76V / 2 = 0.88V x 0.707 ≈ 0.62V

- For R3: Vp = 2.73V / 2 = 1.365V x 0.707 ≈ 0.96V

- For R1 + R2: Vp = 4.27V / 2 = 2.135V x 0.707 ≈ 1.51V

- For R2 + R3: Vp = 4.49V / 2 = 2.245V x 0.707 ≈ 1.58V

Figure 8: Input Voltage Signal

Scale for all: F: 200hz, time period: 5ms, using 0.5ms per box/div

Figure 9: Voltage Signal across R1

Figure 10: Voltage Signal across R2

Figure 11: Voltage Signal across R3

Figure 12: Voltage Signal across R1 and R2

Figure 13: Voltage Signal across R2 and R3

Circuit Diagram:

Post Lab Questions

- Define Ohm’s law?

Ans: Ohm’s law states that the current flowing through a circuit is directly proportional to the applied voltage across that circuit provided the physical conditions remain the same.

V = I.R

- What is resistor?

Ans: A resistor is a device which provide opposition to the flow of current in a circuit.

- How resistor works?

Ans: Resistor converts the electrical energy to the heat energy by the collision of electron.

- What is your inference from the input and output waveforms on oscilloscope?

Ans: This is the condition in which the output voltage signal maintains the same frequency and phase as that of the input signal. But with a reduce in the amplitude of a that signal.

- Show calculations of 7V input signal (Figure 10) from signal generator.

a. Voltage calculations (Following volt/dev and number of boxes of oscilloscope)

b. Frequency/Period calculations (Following Steps in frequency measurements section)

c. Current calculation (Ohm Law)

Ans:

a. Voltage calculations

- Total Vp-p: 7V

- Volts/Div setting: 1V/div

- Number of boxes (vertical): Since Vp-p is 7V, the signal will span 7 vertical divisions (from -3.5 to +3.5).

b. Frequency/Period calculations

- Frequency (f): 200 Hz

- Period (T): T = 1/200 = 5ms

- Time/Div setting: 1ms/div

- Number of boxes (horizontal): 5ms/div, 1ms/div = 5 divisions for one full cycle.

c. Current calculation (Ohm’s Law)

- Total Resistance (Rtotal): 13.1kΩ

Using Ohm's Law for peak current (Ip):

- Vp = V(p-p) / (2) = (7V) / (2) = 3.5V

- Ip = (Vp)(RT) = (3.5V) (13,100Ω) ≈ 0.267mA or 267muA