UNIVERSITY OF ENGINEERING AND TECHNOLOGY

LAHORE CAMPUS

Lab #3

Circuit Analysis Lab

PRESENTED BY:

Muhammad Ibrahim Kaleem – 2025-CE-103

PRESENTED TO:

Prof. Mehmood ul Haq

COMPUTER ENGINEERING – SECTION B

Lab Tasks:

- Measure Current between R1 and R2, and Voltage across R1, R2.

Practical Values:

| R1 Voltage | R2 Voltage | Current |
| --- | --- | --- |
| 3.04V | 1.96V | 0.60mA |

Theoretical Values:

| R1 Voltage | R2 Voltage | Current |
| --- | --- | --- |
| 0.59mA x 5.1kΩ = 3.04V | 0.59mA x 3.3kΩ = 1.96V | 0.595mA |

Total Resistance: 5.1kΩ + 3.3kΩ = 8.4kΩ

Explanation:

Task 1: 5V Series Circuit Implementation

To build the first circuit, we connected a 5V DC power supply in a single loop with two resistors, R1 (5.1kΩ) and R2 (3.3kΩ). The assembly followed a series configuration where the positive terminal of the source connected to the start of R1, the end of R1 connected to the start of R2, and the circuit was completed by connecting the end of R2 back to the negative terminal. To measure the practical R1 and R2 voltages, the Digital Multi-meter (DMM) was set to the DC voltage function and the probes were touched in parallel across each resistor while the power was on. For the current measurement, the circuit was physically opened between the two resistors, and the DMM (set to the mA port and function) was inserted in series into the break so that the total current flowed through the meter. Theoretical calculations using Ohm's Law (I =V/RT) provided a total current of 0.595mA, with voltage drops of 3.04V across R1 and 1.96V across R2

Proteus Diagram:

- Measure Current at node 2 and 3 and Voltage across R1, R2, R3.

Practical Values:

| R1 Voltage | R2 Voltage | R3 Voltage | Current at node 2 | Current at node 3 |
| --- | --- | --- | --- | --- |
| 1.50V | 5.00V | 2.50V | 0.50mA | 0.50mA |

Theoretical Values:

| R1 Voltage | R2 Voltage | R3 Voltage | Current at node 2 | Current at node 3 |
| --- | --- | --- | --- | --- |
| 0.5mA x 3kΩ = 1.5V | 0.5mA x 10kΩ = 5.0V | 0.5mA x 5kΩ = 2.5V | 0.5mA | 0.5mA |

Task 2: 9V Series Circuit Implementation

The second task involved constructing a series circuit using a 9V source and three resistors: R1 (3kΩ), R2 (10kΩ), R3 (5kΩ). We assembled these components end-to-end on a breadboard to ensure a single path for current flow. Voltage measurements for R1, R2, and R3 were obtained by placing the DMM probes across each individual component in a parallel connection. To verify the current at Node 2 and Node 3, the circuit was broken at those specific points and the ammeter was inserted in series; as expected in a series circuit, the current remained constant at both nodes. Theoretically, the total resistance of 18kΩ resulted in a uniform circuit current of 0.5mA, producing individual voltage drops of 1.5V, 5.0V, and 2.5V respectively.

Proteus Diagram:

Post Lab Questions:

- Define energy.

Energy is the capacity to do work.

- Define power.

Power is the rate at which work is done or energy is transferred.

- What is charge?

Charge is the basic electrical property of matter responsible for electric forces, measured in coulombs.

- What is circuit?

An electrical circuit is a closed path through which electric current flows.

- What is the role of a resistor in the series circuit?

A resistor in a series circuit limits current and divides voltage among components.