UNIVERSITY OF ENGINEERING AND TECHNOLOGY

LAHORE CAMPUS

Lab #1

Circuit Analysis Lab

PRESENTED BY:

Muhammad Ibrahim Kaleem – 2025-CE-103

PRESENTED TO:

Prof. Mehmood

COMPUTER ENGINEERING – SECTION B

# Post Lab Questions

1. Define Oscilloscope and write its applications.

An oscilloscope is an electronic instrument used to display and measure electrical signals (voltage) as waveforms over time on a screen.

Applications:

- Observing and analysing signal waveforms.

- Measuring voltage, frequency, and time period.

- Testing and troubleshooting electronic circuits.

2. What is Peak to Peak Voltages?

Peak-to-peak voltage is the difference between the maximum positive peak and the maximum negative peak of a voltage waveform.

Formula:
V(pp) = V(max) – V(min)

It represents the total voltage swing of the signal from the highest point to the lowest point.

3. What is RMS value?

The RMS value of an alternating voltage or current is the effective value that produces the same heating effect as an equivalent DC value.

For a sinusoidal waveform:
V(rms) = (V(max)) / sqrt (2)

It represents the effective or useful value of AC voltage or current.

4. Write the RMS values of Sine, Square and Triangular wave?

RMS values of common waveforms (in terms of peak value (V_(max))):

- Sine Wave:
V(rms) = (V(max)) / sqrt (2)

- Square Wave:
V(rms) = V(max)

- Triangular Wave:
V(rms) = V(rms) = (V(max)) / sqrt (3)

These values represent the effective (DC-equivalent) voltage of each waveform.

5. Explain DC offset?

DC offset is the amount by which an AC signal is shifted above or below the zero-volt (reference) level due to a DC component.

In simple terms, it is the DC voltage added to an AC waveform, causing the waveform to move upward or downward from the centre (0 V) line.