UNIVERSITY OF ENGINEERING AND TECHNOLOGY

LAHORE CAMPUS

Lab #6

Circuit Analysis Lab

PRESENTED BY:

Muhammad Ibrahim Kaleem – 2025-CE-103

PRESENTED TO:

Prof. Mehmood

COMPUTER ENGINEERING – SECTION B

Lab Tasks:

Task 1:

To verify Kirchhoff’s current law and Kirchhoff’s voltage law for the given Circuit.

| Sr. No. | Voltage E | Current |  |  | I1 = I2 + I3 |
| --- | --- | --- | --- | --- | --- |
|  |  | I1 | I2 | I3 |  |
|  | Volt | mA | mA | mA | mA |
| 1. | 5 | 5.68 | 3.12 | 2.56 | 5.68 |
| 2. | 15 | 17.1 | 9.37 | 7.68 | 17.1 |
| 3. | 25 | 28.4 | 15.6 | 12.8 | 28.4 |

General Diagram/ 5V Diagram:

Explanation:

To verify Kirchhoff’s Current Law (KCL), we constructed a parallel network in Proteus using a Regulated Power Supply (RPS) and three resistors: 330Ω, 1kΩ, and 220Ω. We placed the 330Ω resistor in the main branch and connected the 1kΩ and 220Ω resistors in parallel to form a junction (node). Three DC Ammeters, set to the milliamp (mA) range, were inserted into the circuit: one to measure the total entering current ($I_1$) and two to measure the individual branch currents I2 and I3. We systematically varied the RPS voltage from 5V to 25V and recorded the readings. The simulation successfully demonstrated that the total current entering the node I1 was exactly equal to the sum of the currents leaving it I2 + I3, thereby verifying KCL.

Task 2:

| Sr. No. | RPS | Voltage |  |  | E1 = V1 + V2 |
| --- | --- | --- | --- | --- | --- |
|  | E1 | V1 | V2 | V3 |  |
|  | Volt | V | V | V | V |
| 1. | 5 | 3.23 | 1.77 | 1.77 | 5 |
| 2. | 15 | 9.70 | 5.30 | 5.30 | 15 |
| 3. | 25 | 16.2 | 8.83 | 8.83 | 25 |

Explanation:

To verify Kirchhoff’s Voltage Law (KVL), we created a series-parallel hybrid circuit as shown in the manual. The circuit consisted of a 330Ω resistor R1 in series with a parallel combination of a 1kΩ resistor R2 and a 220Ω resistor R3. We utilized three DC Voltmeters to measure the potential difference across each component V1, V2, and V3. After connecting a Ground terminal for reference, we adjusted the RPS voltage E1 between 5V and 15V. We observed that V2 and V3 remained identical due to their parallel connection, and the sum of the voltage drops V1 + V2 consistently equalled the total source voltage E1. This verified that the algebraic sum of voltages in the loop is zero, confirming KVL.

General Diagram/ 5V Diagram:

Proved:

Hence proved KCL and KVL from the given circuits.

Post Lab Questions

Define Kirchhoff’s current law.

The total current entering a junction (node) is equal to the total current leaving the junction.

Define Kirchhoff’s voltage law.

The sum of all voltages around a closed loop in a circuit is zero.

What are the applications of regulated power supply?

- Provides constant voltage to electronic devices

- Used in computers, TVs, and mobile chargers

- Protects circuits from voltage fluctuations

- Essential in labs for testing electronic circuits

What are the limitations of KCL?

- Not accurate at very high frequencies (due to charge storage effects)

- Assumes no energy is stored at the node (ignores capacitance)

- Becomes complex in large circuits with many nodes